# 88-91 DX to Si Harness Connections Wiring Harness

Step 1. Route plug adapter harness through the fire wall to the ECU.

Step 2. Find orange and white wires at C1 and C2.

Remove white retaining clips from harness plug and remove the pins/wires from this location.

Step 3. Find locations B10 and B12 (in same models, B12 may have a pin/wire already-remove if found). Place pin from C1 into location B10 and wire from C2 into location B12. This will now leave C1 and C2 empty, but these locations will be used shortly.

Step 4. Add wires in this order

| Wire Color | ECU connection |
| --- | --- |
| Red | A3 |
| Yellow | A7 |
| Green | C1 |
| Blue | C2 |

Step 5. Install wiring on engine and plug in the clips as marked. If markings have fallen off, the wiring will normally only be long enough to make it to one correct location.

ECU Diagram