Recommendations for your new engine –LONG BLOCK

Normally a new pilot bearing (flywheel) is needed.

Throw out bearing (your stock transmission)

TPS is normally broke/missing, your US factory one will be used.

Cap/Rotor and Plugs

Plug wires

Valve cover gasket and oil pan gasket

Timing belt and water pump –if this set is done, change the cam seals

Any drive belts

This engine was probably purchased because of the known lower mileage with JDM engines. It is still a recommendation but not a requirement to do some or all of the steps above as it will be much easier with the engine out of the car.

The below is required

ALWAYS use fresh oil and coolant. Drain ALL fluid from engine and replenish prior to starting.

Change oil and filter in 500 miles. Again at 2,000

Change/flush coolant at 2,000

***Any extra accessories (alternator, AC pump, etc) are that –extra. These are NOT covered under any warranty.

If you find yourself with questions, please email us at tech@jdmcars.com or visit our website at www.jdmcars.com