Recommendations for your new engine swap –

TPS is normally broke/missing, your US factory one will be used.

This engine was probably purchased because of the known lower mileage with JDM engines. It is still a recommendation but not a requirement to do some or all of the steps below as it will be much easier with the engine out of the car.

Cap/Rotor and Plugs

Plug wires

Valve cover gasket and oil pan gasket

Timing belt and water pump –if this is done, change the cam seals

Any drive belts

Inspect the clutch. Inspect all bearing and main seal –replace if needed. Inspect/replace the axel seals.

Below are requirements

ALWAYS use fresh oil and coolant. Drain ALL fluid from engine and replenish prior to starting. This is for the motor and transmission.
It is also a good idea to flush the motor oil through the engine.

Change motor oil and filter in 500 miles. Again at 2,000

Change/flush coolant at 2,000

***Any extra accessories (alternator, AC pump, etc) are that –extra. These are NOT covered under any warranty.

If you find yourself with questions, please email us at tech@jdmcars.com or visit our website at www.jdmcars.com